PART A

| #### LOCATION DETAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  |  |  |  |  | ELR |  |  |  |  | Line(s) |  |  |  | Track ID(s) |  |  |  |  |  |
| Mileage From |  |  | m |  |  |  |  |  | yds |  |  | Mileage To |  |  |  | m |  |  | yds |  |
| Published speed (PSR) |  | mph |  | Site protected by ESR/TSR? |  |  |  |  |  | ##### Yes / No |  |  |  | If yes, what speed? |  |  |  |  |  | mph |
| Planned opening line speed |  | mph |  | Note: |  |  | if planned opening speed is higher than existing PSR/TSR/ESR, TEF3203 is also to be completed by an Authorised Person (see NR/L2/TRK/001/mod13) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| #### WORK TYPE |  |  |  |
| --- | --- | --- | --- |
| The type of work carried out determines which part of this TEF requires to be completed |  |  |  |
| Work undertaken |  | ##### Part to be completed |  |
| - Plain line tamping; measurement and compensation method | ##### Yes | ##### Part B |  |
| - Plain line tamping; applying a design | ##### Yes | ##### Part B |  |
| - Plain line tamping; absolute track geometry (ATG) location | ##### Yes | ##### LNW ATG forms to be used |  |
| - DTS | ##### Yes | ##### Part B |  |
| - S&C tamping; single machine | ##### Yes | ##### Part B (for each affected TID) |  |
| - S&C tamping; multiple machines | ##### Yes | ##### Part B (for each affected TID) |  |
| - Plain line stoneblowing | ##### Yes | ##### Part B |  |
| - S&C stoneblowing with multi-purpose stoneblower | ##### Yes | ##### Part C (for each affected TID) |  |

| #### SIGN OFF |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part |  |  | completed by (name): |  |  |  |  |  |  |  |  |  |
| Date of inspection: |  |  |  |  |  | Time of inspection: |  |  |  |  |  |  |
| Following inspection of the track and a review of the post-work recording run data, I can confirm that: - track geometry (twist, top, line and gauge) is in accordance with ATG requirements / installation tolerances / maintenance tolerances.# structural and passing clearances have not been affected / have been checked and are within tolerance . # - the track is safe to re-open for trains at mph # delete as required |  |  |  |  |  |  |  |  |  |  |  |  |
| For ATG routes |  | Tamping Leader (name) |  |  |  |  |  | Signed |  |  | Date |  |
|  |  | TGS (name) |  |  |  |  |  | Signed |  |  | Date |  |
| All other routes |  | TGS (name) |  |  |  |  |  | Signed |  |  | Date |  |

| #### ADDITIONAL WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The following additional work is required: (Include a timescale for completion where appropriate) |  |  |  |  |  |
| I have reviewed the proposed follow-up work and taken the necessary actions. Work entered into ellipse? Yes / No |  |  |  |  |  |
| Name of SM[T]: |  | Signed: |  | Date: |  |

PART B:

Tamper or DTS (plain line or S&C) or Stoneblower (plain line)

| #### PRE-WORK SITE RECORD |  |  |  |
| --- | --- | --- | --- |
| Pre work site record |  |  |  |
| Maximum recorded 35m vertical SD value |  | Maximum recorded 35m lateral SD value |  |

| #### WORK-IN-PROGRESS CHECKS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Checks are particularly critical - in the first 40-50 yards (to confirm OTM working correctly) - at run-in and run-out locations - adjacent to, on or under structures |  |  |  |  |  |  |
| Location $ | Design cant | Actual cant | Error [+/-] | Vertical datum [+/-] | Structure clearance | Passing clearance |
| $ use mileage, chainage interval, half-chord or sleeper count |  |  |  |  |  |  |

| #### POST-WORK SITE RECORD & CHECKLIST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Post work site record |  |  |  |  |  |  |  |  |  |  |
| Maximum Twist |  |  | mm | Maximum recorded 35m vertical SD value |  |  |  |  |  |  |
| Maximum cross level error |  |  | mm | Maximum recorded 35m lateral SD value |  |  |  |  |  |  |
| For measurement & compensation method |  | Amount of lift applied |  |  |  | mm | Amount of slue applied |  |  | mm |
| Checklist |  |  |  |  |  |  |  | True? (see Note 1) |  |  |
| Track design | Designed with lifts and slues that comply with design limits for the opening speed |  |  |  |  |  |  | Yes / No |  |  |
| Twist | Track is free of 3m twists that equal or exceed 10mm (1 in 300 or worse) |  |  |  |  |  |  | Yes / No |  |  |
| Cross level | Track is within maintenance tolerance for the planned opening speed (see NR/L2/TRK/001/mod12) |  |  |  |  |  |  | Yes / No |  |  |
| Vertical track geometry | Peak running 35m vertical SD is below ‘Maximum band’ value for the planned opening speed over the full extent of the delivered maintenance. (see NR/L2/TRK/001/mod11) |  |  |  |  |  |  | Yes / No |  |  |
| Lateral track geometry | Peak running 35m lateral SD is below ‘Maximum band’ value for the planned opening speed over the full extent of the delivered maintenance. (see NR/L2/TRK/001/mod11) |  |  |  |  |  |  | Yes / No |  |  |
| Voiding | Any voiding is within maintenance limits. Particularly critical in S&C if tracks are lifted independently (one machine) |  |  |  |  |  |  | Yes / No |  |  |
| Ballast | Ballast is topped-up & boxed-in to full profile |  |  |  |  |  |  | Yes / No |  |  |
| S+T equipment | S+T components are free from damage and securely attached |  |  |  |  |  |  | Yes / No / N/A |  |  |
| Adjustment switches | Adjustment switch rail tips and cut-outs are correctly supported, and rail straps are secure (see NR/L2/TRK/001/mod16) |  |  |  |  |  |  | Yes / No / N/A |  |  |
| Conductor Rail | Checked: sitting correctly within the lugs, ramp ends checked, conductor rail joints intact and level, no cable damage, damaged or missing pots, guard boards re-instated and in gauge, clear of ballast |  |  |  |  |  |  | Yes / No / N/A |  |  |
| Overhead Line Electrification Equipment | Check bonds and bond connections to track are not damaged |  |  |  |  |  |  | Yes / No / N/A |  |  |
|  | Have planned OLE adjustments and measurements been completed? |  |  |  |  |  |  | Yes / No / N/A |  |  |
| Design Implementation | Was a design implemented by someone other than the TQS? |  |  |  |  |  |  | Yes / No / N/A |  |  |
|  | If Yes, Name: |  |  |  | Post: |  |  | Signed: |  |  |
| Note 1: If a ‘NO’ answer is recorded (an unsatisfactory condition), then (add details below): - consider mitigation required (which may include the imposition of an ESR) and - identify any additional works required. |  |  |  |  |  |  |  |  |  |  |

#### DETAILS OF POST WORK MITIGATION / ADDITIONAL WORKS

Comments: (Add details of any unsatisfactory conditions, mitigation or additional works required)

| #### SIGN OFF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date of inspection: |  |  | Time of inspection: |  |  |  |
| I have inspected the track at the above location, reviewed the output from the OTM and confirm that: - all checks are satisfactory and the track is safe to re-open at the planned opening speed of mph. OR - mitigation has been applied (as detailed above) and the track is safe to re-open at a speed of mph. OR - the track is safe to re-open for trains at a reduced speed of mph. |  |  |  |  |  |  |
| TGS (name): |  |  |  | Signed: |  |  |

PART C: S&C Stoneblower

| #### SITE CHECKS / INSPECTION |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Planned opening speed |  |  |  | mph |  | * “within S&C” defined as between the switch front and last through bearer. |  |  |  |  |  |
| Ramp length* |  |  | 5 metre short ramp within S&C |  |  |  |  | ##### Yes | Limit opening speed to 30mph |  |  |
|  |  |  | 10 metre short ramp within S&C |  |  |  |  | ##### Yes | Limit opening speed to 60mph |  |  |
|  |  |  | 35 metre min. standard ramp |  |  |  |  | ##### Yes | No limit to opening speed |  |  |
| Post work site record |  |  |  |  |  |  |  |  |  |  |  |
| Maximum Twist |  |  |  |  | mm | Maximum recorded 35m vertical SD value |  |  |  |  |  |
| Maximum cross level error |  |  |  |  | mm | Maximum recorded 35m lateral SD value |  |  |  |  |  |
| Remarks: (Add further details, as applicable) |  |  |  |  |  |  |  |  |  |  |  |
| Checklist though out tamp site, including run in and out (opening at 90mph or less) |  |  |  |  |  |  |  |  |  | True? (see Note 1) |  |
| Track design |  | Designed with lifts and slues that comply with design limits for the opening speed |  |  |  |  |  |  |  | Yes / No |  |
| Component integrity, rejection and voiding |  | Track ‘lifted as one’, minimal rejection and no significant voiding under the passage of the MPSB |  |  |  |  |  |  |  | Yes / No |  |
| Twist |  | Track is free of 3m twists that equal or exceed 15mm (1 in 200 or more severe) |  |  |  |  |  |  |  | Yes / No |  |
| Cross level |  | Track is within maintenance tolerance for the planned opening speed (see NR/L2/TRK/001/mod12) |  |  |  |  |  |  |  | Yes / No |  |
| Vertical track geometry |  | Peak running 35m vertical SD is below ‘Maximum band’ value for the planned opening speed over the full extent of the delivered maintenance. (see NR/L2/TRK/001/mod11) |  |  |  |  |  |  |  | Yes / No |  |
| Lateral track geometry |  | Peak running 35m lateral SD is below ‘Maximum band’ value for the planned opening speed over the full extent of the delivered maintenance. (see NR/L2/TRK/001/mod11) |  |  |  |  |  |  |  | Yes / No |  |
| Ballast |  | Ballast is topped-up & boxed-in to full CWR profile (including plain line for a distance of 50m each side of S&C) |  |  |  |  |  |  |  | Yes / No |  |
| Lateral Resistance Plates (LRPs) |  | Where LRPs are fitted, these have not been disturbed during the completion of the works. |  |  |  |  |  |  |  | Yes / No |  |
| S+T equipment |  | S+T components are free from damage and securely attached |  |  |  |  |  |  |  | Yes / No / N/A |  |
| Adjustment switches |  | Adjustment switch rail tips and cut-outs are correctly supported, and rail straps are secure (see NR/L2/TRK/001/mod16) |  |  |  |  |  |  |  | Yes / No / N/A |  |
| Conductor Rail |  | Checked: sitting correctly within the lugs, ramp ends checked, conductor rail joints intact and level, no cable damage, damaged or missing pots, guard boards re-instated and in gauge, clear of ballast |  |  |  |  |  |  |  | Yes / No / N/A |  |
| Overhead Line Electrification Equipment |  | Check bonds and bond connections to track are not damaged |  |  |  |  |  |  |  | Yes / No / N/A |  |
|  |  | Have planned OLE adjustments and measurements been completed? |  |  |  |  |  |  |  | Yes / No / N/A |  |
| Design Implementation |  | Was a design implemented by someone other than the TQS? |  |  |  |  |  |  |  | Yes / No / N/A |  |
|  |  | If Yes, Name: |  |  |  |  | Post: |  |  | Signed: |  |
| Note 1: If a ‘NO’ answer is recorded (an unsatisfactory condition), then the opening speed shall be limited to 60mph. Add comments below (including details of any additional mitigation to be applied.) |  |  |  |  |  |  |  |  |  |  |  |
| Checklist (opening above 90mph) |  |  |  |  |  |  |  |  |  |  | True? (see Note 2) |
| Twist | Peak 3m Twist (obtained from the post-work measure processed data) does not exceed 10mm. |  |  |  |  |  |  |  |  |  | Yes / No |
| Vertical track geometry | Peak 35m Top (obtained from the post-work measure processed data) does not exceed 12mm. |  |  |  |  |  |  |  |  |  | Yes / No |
| Ballast | Ballast is topped-up & boxed-in to full CWR profile (including plain line for a distance of 100m each side of S&C). |  |  |  |  |  |  |  |  |  | Yes / No |
| Lateral Resistance Plates (LRPs) | Lateral resistance plates are fitted and secure on timber bearers carrying BV, CV and DV switches at line speed ≥ 100mph. Where LRPs are fitted (irrespective of switch size or speed), these have not been disturbed during the completion of the works. |  |  |  |  |  |  |  |  |  | Yes / No |
| Note 2: If a ‘NO’ answer is recorded (an unsatisfactory condition), then the opening speed shall be limited to 90mph. Add comments below (including details of any additional mitigation to be applied.) |  |  |  |  |  |  |  |  |  |  |  |
| #### DETAILS OF POST WORK MITIGATION / ADDITIONAL WORKS |  |  |  |  |  |  |  |  |  |  |  |
| Comments: (Add details of any unsatisfactory conditions, mitigation or additional works required) |  |  |  |  |  |  |  |  |  |  |  |

| #### SIGN OFF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date of inspection: |  |  | Time of inspection: |  |  |  |
| I have inspected the track at the above location, reviewed the output from the OTM and confirm that: - all checks are satisfactory and the track is safe to re-open at the planned opening speed of mph. OR - the track is safe to re-open for trains at a reduced speed of mph. |  |  |  |  |  |  |
| TGS (name): |  |  |  | Signed: |  |  |
| Countersigned (mentoring or planned TSR/PSR above 90mph) |  |  |  |  |  |  |
| Name: |  |  |  | Post: |  |  |
| Signed: |  |  |  |  |  |  |